[image: image1.jpg]



Licitación Pública Regional No. 32065001-041-12

Anexo C

ANEXO C
Módulo de conectividad Fiber Channel
Cantidad: 02

Marca Aceptada:

Brocade
Características Mínimas

· Switch para chasis blade, 8Gb con auto negociación para 2, 4 y 8 Gb/s

· Bandwidth de 384GB/sec. end to end.

· Deberá de ser compatible con el equipo solicitado en la partida 2
· Capacidad de 8 puertos externos y 16 puertos internos.

· Deberá de soportar la conectividad en cascada

· Capacidad opcional de soportar Advanced Trunking

· Fuentes de alimentación y abanicos del tipo hot-plug, soportador por el equipo solicitado en la partida 2.
· Chasis embebido dentro del equipo solicitado en la partida 2.

· Capacidad de soportar Trunking y Zoning software
· Switch core: Non-blocking

· Fabric Latency: 700 nSec.

· Maximum frame size: 2112 byte payload

· Classes of service: class2, class 3, class F (Inter-Switch frames)

· Port Types: FL_Port, F_Port y E_Port, auto-descubrimiento basado en tipo de switch (U_Port)
· Soporte de Unicast y Broadcast
· Media types: Hot-pluggable, B-Series Branded Small Form Factor Pluggable (SFP), LC connector; Short-Wave Laser (SWL), Long-Wave Laser (LWL); Extended Long-Wave Laser (ELWL); distance depends on fiber-optic cable and port speed. Supports SFP (4/2/1 Gbit/sec) and/or SFP+ (8/4/2 Gbit/sec) optical transceivers.

· Short-wave, long-wave, and extended long wave laser transceivers (SFP's) are supported.

· Fabric Services: Simple Name Server, Registered State Change Notification (RSCN). Includes Advanced Web Tools Enhanced group Management and Advanced Zoning. Optional fabric services include: Adaptive Networking, Fabric Watch, ISL Trunking, Advanced Performance Monitoring, and Extended Fabrics.

· Opciones de:SFP media, port upgradeable licensing, advanced management software licensing

· Software de soporte: Telnet, SNMP (FE MIB, FC Management MIB), HP Systems Insight Manager, HP Storage Essentials, HP Onboard Administrator for BladeSystem, Advanced WebTools, Enhanced Group

· Management, Advanced Zoning, SAN Network Advisor, Fabric Watch, and Advanced Performance Monitoring

· Management Access: Web Tools, SMI-s provider, SNMP, Telnet, and Secure Telnet

· Diagnostics: Diagnostics POST and embedded online/offline diagnostics

· Fiber Channel Standards and Revisions:

· FC-AL-2 INCITS 332: 1999

· FC-GS-5 ANSI INCITS 427: (includes the following.)

· FC-GS-4 ANSI INCITS 387: 2004

· FC-IFR INCITS 1745-D, revision 1.03 (under development)

· FC-SW-4 INCITS 418:2006 (includes the following)

· FC-SW-3 INCITS 384: 2004

· FC-VI INCITS 357: 2002

· FC-TAPE INCITS TR-24: 1999

· FC-DA INCITS TR-36: 2004 (includes the following)

· FC-FLA INCITS TR-20: 1998

· FC-PLDA INCIT S TR-19: 1998

· FC-MI-2 ANSI/INCITS TR-39-2005

· FC-PI INCITS 352: 2002

· FC-PI-2 INCITS 404: 2005

· FC-PI-4 INCITS 1647-D, revision 7.1 (under development)

· FC-FS-2 ANSI/INCITS 424:2006 (includes the following)

· FC-FS INCITS 373: 2003

· FC-LS INCITS 433: 2007

· FC-BB-3 INCITS 414: 2006 (includes the following)

· FC-BB-2 INCITS 372: 2003
· FC-SB-3 INCITS 374: 2003 (replaces FC-SB ANSI X3.271: 1996; FC-SB-2 INCITS 374:

2001)

· RFC 2625 IP and ARP Over FC

· RFC 2837 Fabric Element MIB

· MIB-FA INCITS TR-32: 2003

· FCP-2 INCITS 350: 2003 (replaces FCP ANSI X3.269: 1996)

· SNIA Storage Management Initiative Specification (SMI-S) Version 1.2 (includes the

following)

· SNIA Storage Management Initiative Specification (SMI-S) Version 1.03 ISO standard

· IS24775-2006. Replaces (ANSI INCITS 388: 2004)

· SNIA Storage Management Initiative Specification (SMI-S) Version 1.1.0

· SNIA Storage Management Initiative Specification (SMI-S) Version 1.2.0
· Garantía de línea del fabricante por el período mínimo de 3 años (7 X 24 X 365), con un tiempo de respuesta de 6 a 8 horas para atender la falla en sitio y un tiempo de reparación del equipo al día siguiente en caso de que se requiera el reemplazo de una pieza, a partir de que se haya levantado el reporta al número 01-800.

